FUNCTION; PIN; WIRE COLOR. Left Rear Door Spkr +; 5; WT/VT. Left Rear Door Spkr -; 6; GY/WT. Right Rear Door Spkr +; 7; WT (WT/VT at speaker). Right Rear Door Spkr -; 8; GY (GY/WT at speaker). Left Front Door Spkr +; 9; WT/BK (WT at speakers). Left Front Door Spkr -; 10; GY/BK (GY at speakers). Right Front Door Spkr +; 11; WT/RD (WT at speakers). Right Front Door Spkr -; 12; GY/RD (GY at speakers). Right Rear - Input; 15; WT/BU. Right Rear + Input; 16; YW/GN. Left Rear - Input; 17; YW/BU. Left Rear + Input; 18; BN/BU. Right Front - Input; 19; GN. Right Front + Input; 20; RD. Left Front - Input; 21; L.GN. Left Front + Input; 22; VT. Amp Remote; 23; BU.

FUNCTION; PIN; WIRE COLOR. Constant 12 Volt; 25; RD. Subwoofer Spkr -; 26; GY/WT. Chassis Ground; 29; BK. Subwoofer Spkr +; 30; WT/BU.

Read numbers from pin side of vehicle connector. The front door tweeters are wired in parallel with the front door woofers.